Auto-ignition Characteristics of Gasoline and Diesel Fuel Blends: A High-Pressure Ignition Delay and Kinetic Modelling Study

Yang Li,1,2

1Science and Technology on Combustion, Internal Flow and Thermo­structure Laboratory, Northwestern Polytechnical University, Xi'an 710072, China

2Clean Combustion Research Centre, King Abdullah University of Science and Technology, Thuwal, Saudi Arabia

Supplementary Material 1

Table S1. List of literature studies on autoignition of diesel blends [1].

| Study | Facility | Pressure (atm) | Temperature (K) | φ | Oxidizer |
| --- | --- | --- | --- | --- | --- |
| Spadaccini et al. [2] | Flow reactor | 10 – 30 | 625 – 1000 | 0.3 – 1.0 | Air |
| Penyazkov et al. [3] | Shock tube | 4.7 – 10.4 | 1065 – 1838 | 0.5 – 2.0 | Air |
| Haylett et al. [4] | Shock tube | 1.7 – 8.6 | 947 – 1261 | 0.2 – 1.0 | Varying O2/Ar ratios |
| Gowdagiri et al. [5] | Shock tube | 10, 20 | 671 – 1266 | 0.5, 1.0 | Air |

Table S2. Composition of Coryton gasoline.

| Name | Content (mol %) | Name | Content (mol %) |
| --- | --- | --- | --- |
| n-butane | 1.44 | 2-methylheptane | 10.61 |
| iso-butane | 0.26 | ethyl-cyclohexane | 2.16 |
| 1-butene | 0.18 | p-xylene | 5.05 |
| n-pentane | 2.90 | n-nonane | 0.16 |
| iso-pentane | 5.33 | 2-methyloctane | 2.03 |
| 1-pentene | 5.43 | propyl-benzene | 2.52 |
| cyclopentane | 0.78 | n-decane | 0.30 |
| n-hexane | 0.96 | 2-methylnonane | 0.34 |
| 2-methylpentane | 3.16 | butyl-cyclohexane | 0.58 |
| 1-hexene | 0.91 | butyl-benzene | 2.26 |
| cyclohexane | 0.65 | n-undecane | 0.57 |
| benzene | 0.30 | 2-methyldencane | 1.17 |
| n-heptane | 1.61 | 1-methyldecalin | 0.33 |
| 2-methylhexane | 5.88 | 1-methyl,3-butyl-benzene | 1.79 |
| 1-heptene | 1.22 | 2-methylundecane | 0.10 |
| methylcyclohexane | 1.16 | dimethyltetralin | 0.61 |
| toluene | 16.54 | ethanol | 4.24 |
| n-octane | 0.07 | - | - |

Table S3. Composition of Coryton diesel.

| Name | Content (mol %) | Structure |
| --- | --- | --- |
| 1-hexadecene | 2.98 |  |
| dodecylcyclohexane | 14.73 |  |
| n-dodecane | 25.16 |  |
| heptamethylnonane | 19.72 |  |
| naphthalene | 31.16 |  |
| 2,3,4-trimethylpentane | 6.25 |  |

# Coryton gasoline/diesel 75/25 IDT experimental data:

  - HPST data

| p5 [atm] | T5 [K] | 1000/T5 | tau [μs] |
| --- | --- | --- | --- |
| 9.7 | 1189 | 0.84 | 367.3 |
| 9.6 | 1088 | 0.92 | 1193.0 |
| 9.0 | 1248 | 0.80 | 221.1 |
| 8.9 | 1007 | 0.99 | 2436.0 |
| 9.7 | 1054 | 0.95 | 1625.0 |
| 9.7 | 1140 | 0.88 | 704.4 |
| 9.4 | 1364 | 0.73 | 76.4 |
| 9.6 | 1331 | 0.75 | 107.6 |

Table S4. 0.97% Coryton gasoline/diesel 75/25, 20.79% O2, 78.24% N2, φ = 0.5 in air, p5 ≈ 10 atm.

| p5 [atm] | T5 [K] | 1000/T5 | tau [μs] |
| --- | --- | --- | --- |
| 18.7 | 1074 | 0.93 | 895.8 |
| 19.7 | 1199 | 0.83 | 212.5 |
| 19.8 | 1150 | 0.87 | 354.8 |
| 19.7 | 1048 | 0.95 | 1128.0 |
| 19.8 | 1001 | 1.00 | 1784.0 |
| 19.1 | 1233 | 0.81 | 146.6 |
| 19.0 | 1277 | 0.78 | 88.3 |
| 19.0 | 1324 | 0.76 | 55.0 |
| 19.9 | 953 | 1.05 | 2760.0 |
| 19.2 | 1095 | 0.91 | 682.0 |

Table S5. 0.97% Coryton gasoline/diesel 75/25, 20.79% O2, 78.24% N2, φ = 0.5 in air, p5 ≈ 20 atm.

| p5 [atm] | T5 [K] | 1000/T5 | tau [μs] |
| --- | --- | --- | --- |
| 9.4 | 980 | 1.02 | 2670.0 |
| 10.6 | 974 | 1.03 | 2731.0 |
| 10.1 | 1107 | 0.90 | 743.8 |
| 10.0 | 1207 | 0.83 | 250.3 |
| 9.7 | 1292 | 0.77 | 113.2 |
| 10.2 | 1063 | 0.94 | 1106.0 |
| 10.1 | 1160 | 0.86 | 472.8 |
| 9.9 | 1349 | 0.74 | 67.1 |
| 9.7 | 1013 | 0.99 | 2058.0 |

Table S6. 1.91% Coryton gasoline/diesel 75/25, 20.6% O2, 77.49% N2, φ = 1.0 in air, p5 ≈ 10 atm.

| p5 [atm] | T5 [K] | 1000/T5 | tau [μs] |
| --- | --- | --- | --- |
| 19.2 | 989 | 1.01 | 1450.0 |
| 19.7 | 949 | 1.05 | 2073.0 |
| 19.5 | 1094 | 0.91 | 514.4 |
| 19.5 | 1193 | 0.84 | 183.2 |
| 19.2 | 1283 | 0.78 | 72.8 |
| 19.5 | 1044 | 0.96 | 881.4 |
| 19.6 | 1145 | 0.87 | 304.0 |
| 19.5 | 1241 | 0.81 | 112.4 |
| 19.2 | 910 | 1.10 | 2813.0 |

Table S7. 1.91% Coryton gasoline/diesel 75/25, 20.6% O2, 77.49% N2, φ = 1.0 in air, p5 ≈ 20 atm.

| p5 [atm] | T5 [K] | 1000/T5 | tau [μs] |
| --- | --- | --- | --- |
| 10.1 | 1108 | 0.90 | 647.6 |
| 10.4 | 1019 | 0.98 | 1677.0 |
| 10.2 | 1073 | 0.93 | 904.3 |
| 9.9 | 1199 | 0.83 | 267.9 |
| 10.1 | 1159 | 0.86 | 400.5 |
| 9.7 | 1243 | 0.80 | 187.8 |
| 9.8 | 1296 | 0.77 | 121.9 |
| 9.7 | 1341 | 0.75 | 89.2 |

Table S8. 3.75% Coryton gasoline/diesel 75/25, 20.21% O2, 76.03% N2, φ = 2.0 in air, p5 ≈ 10 atm.

| p5 [atm] | T5 [K] | 1000/T5 | tau [μs] |
| --- | --- | --- | --- |
| 19.2 | 989 | 1.01 | 1071.0 |
| 20.1 | 1107 | 0.90 | 328.2 |
| 19.8 | 1202 | 0.83 | 133.1 |
| 20.1 | 956 | 1.05 | 1425.0 |
| 19.9 | 902 | 1.11 | 2333.0 |
| 20.3 | 1061 | 0.94 | 532.4 |
| 19.6 | 1146 | 0.87 | 232.9 |
| 19.5 | 1243 | 0.80 | 93.3 |
| 19.5 | 1293 | 0.77 | 56.3 |

Table S9. 3.75% Coryton gasoline/diesel 75/25, 20.21% O2, 76.03% N2, φ = 2.0 in air, p5 ≈ 20 atm.

  - RCM data

| Pc [atm] | Tc [K] | 1000/Tc | tau [ms] |
| --- | --- | --- | --- |
| 10.4 | 745 | 1.34 | 31.53 |
| 10.6 | 746 | 1.34 | 31.34 |
| 10.5 | 745 | 1.34 | 35.05 |
| 10.8 | 775 | 1.29 | 39.7 |
| 10.8 | 775 | 1.29 | 39.7 |
| 10.8 | 775 | 1.29 | 38.4 |
| 9.6 | 803 | 1.25 | 52.1 |
| 9.5 | 801 | 1.25 | 57.98 |
| 9.6 | 803 | 1.25 | 45.64 |
| 9.7 | 835 | 1.20 | 49.57 |
| 9.7 | 835 | 1.20 | 49.93 |
| 9.6 | 834 | 1.20 | 50.67 |
| 10.1 | 879 | 1.14 | 20.44 |
| 10.1 | 879 | 1.14 | 21.16 |
| 10.2 | 880 | 1.14 | 21.16 |
| 10.5 | 932 | 1.07 | 5.782 |
| 10.5 | 933 | 1.07 | 6.37 |
| 10.6 | 933 | 1.07 | 4.98 |
| 10.5 | 933 | 1.07 | 6.6 |

Table S10. 1.91% Coryton gasoline/diesel 75/25, 20.6% O2, φ = 1.0 in ‘air’, pc ≈ 10 atm.

| Pc [atm] | Tc [K] | 1000/Tc | tau [ms] |
| --- | --- | --- | --- |
| 20.5 | 746 | 1.34 | 8.53 |
| 20.5 | 746 | 1.34 | 7.494 |
| 20.5 | 746 | 1.34 | 8.45 |
| 20.3 | 772 | 1.30 | 7.35 |
| 21.1 | 773 | 1.29 | 7.41 |
| 21.0 | 773 | 1.29 | 6.77 |
| 21.1 | 773 | 1.29 | 6.87 |
| 20.5 | 802 | 1.25 | 6.81 |
| 20.8 | 805 | 1.24 | 6.78 |
| 20.6 | 803 | 1.25 | 7.11 |
| 21.0 | 837 | 1.19 | 6.04 |
| 20.6 | 836 | 1.20 | 6.24 |
| 20.8 | 838 | 1.19 | 5.50 |
| 20.8 | 838 | 1.19 | 5.53 |
| 20.4 | 881 | 1.14 | 4.41 |
| 20.4 | 881 | 1.14 | 4.19 |

Table S11. 1.91% Coryton gasoline/diesel 75/25, 20.6% O2, φ = 1.0 in ‘air’, pc ≈ 20 atm.

# Coryton gasoline/diesel 50/50 IDT experimental data:

  - RCM data

| Pc [atm] | Tc [K] | 1000/Tc | tau [ms] |
| --- | --- | --- | --- |
| 10.0 | 767 | 1.30 | 63.05 |
| 9.8 | 765 | 1.31 | 60.22 |
| 9.8 | 765 | 1.31 | 64.71 |
| 9.9 | 794 | 1.26 | 62.35 |
| 9.9 | 795 | 1.26 | 58.64 |
| 9.9 | 795 | 1.26 | 56.57 |
| 10.0 | 796 | 1.26 | 65.51 |
| 9.8 | 831 | 1.20 | 68.87 |
| 9.7 | 831 | 1.20 | 66.07 |
| 9.7 | 831 | 1.20 | 57.32 |
| 9.8 | 831 | 1.20 | 60.80 |
| 9.8 | 873 | 1.15 | 50.09 |
| 9.7 | 872 | 1.15 | 51.51 |
| 9.7 | 873 | 1.15 | 50.05 |
| 10.0 | 929 | 1.08 | 18.97 |
| 10.1 | 931 | 1.07 | 19.20 |
| 10.0 | 928 | 1.08 | 18.95 |
| 9.8 | 995 | 1.01 | 4.37 |
| 9.9 | 997 | 1.00 | 4.24 |
| 10.0 | 999 | 1.00 | 4.47 |

Table S12. 0.82% Coryton gasoline/diesel 50/50, 20.83% O2, φ = 0.5 in ‘air’, pc ≈ 10 atm.

| Pc [atm] | Tc [K] | 1000/Tc | tau [ms] |
| --- | --- | --- | --- |
| 20.1 | 768 | 1.30 | 13.88 |
| 19.9 | 766 | 1.31 | 11.96 |
| 20.1 | 768 | 1.30 | 12.11 |
| 20.3 | 799 | 1.25 | 9.67 |
| 20.2 | 798 | 1.25 | 10.68 |
| 20.3 | 798 | 1.25 | 9.67 |
| 19.8 | 833 | 1.20 | 8.90 |
| 19.8 | 833 | 1.20 | 9.86 |
| 19.7 | 833 | 1.20 | 7.55 |
| 20.0 | 835 | 1.20 | 8.54 |
| 20.3 | 837 | 1.19 | 8.94 |
| 19.7 | 874 | 1.14 | 8.54 |
| 19.7 | 874 | 1.14 | 7.97 |
| 19.6 | 874 | 1.14 | 7.92 |
| 20.1 | 878 | 1.14 | 8.34 |
| 19.8 | 875 | 1.14 | 9.05 |
| 20.7 | 935 | 1.07 | 2.71 |
| 20.7 | 935 | 1.07 | 2.76 |

Table S13. 0.82% Coryton gasoline/diesel 50/50, 20.83% O2, φ = 0.5 in ‘air’, pc ≈ 20 atm.

| Pc [atm] | Tc [K] | 1000/Tc | tau [ms] |
| --- | --- | --- | --- |
| 9.5 | 742 | 1.35 | 18.13 |
| 9.5 | 742 | 1.35 | 17.92 |
| 9.5 | 742 | 1.35 | 17.94 |
| 10.1 | 768 | 1.30 | 15.32 |
| 10.2 | 770 | 1.30 | 15.49 |
| 10.0 | 767 | 1.30 | 15.81 |
| 10.0 | 800 | 1.25 | 19.35 |
| 9.9 | 799 | 1.25 | 21.35 |
| 9.9 | 799 | 1.25 | 19.39 |
| 10.1 | 836 | 1.20 | 23.68 |
| 10.0 | 835 | 1.20 | 23.22 |
| 10.0 | 835 | 1.20 | 24.14 |
| 9.9 | 834 | 1.20 | 22.32 |
| 10.1 | 836 | 1.20 | 22.90 |
| 9.9 | 877 | 1.14 | 17.15 |
| 10.0 | 880 | 1.14 | 18.05 |
| 9.9 | 878 | 1.14 | 16.77 |
| 9.8 | 933 | 1.07 | 7.44 |
| 10.0 | 936 | 1.07 | 7.24 |
| 9.9 | 935 | 1.07 | 6.70 |
| 9.9 | 935 | 1.07 | 7.113 |

Table S14. 1.63% Coryton gasoline/diesel 50/50, 20.65% O2, φ = 1.0 in ‘air’, pc ≈ 10 atm.

Reference

[1] G. Kukkadapu, C.-J. Sung, Autoignition study of ULSD#2 and FD9A diesel blends, Combustion and Flame 166 (2016) 45-54.

[2] L.J. Spadaccini, J.A. Tevelde, Autoignition characteristics of aircraft-type fuels, Combustion and Flame 46 (1982) 283-300.

[3] O. Penyazkov, K. Sevrouk, V. Tangirala, N. Joshi. 2009. Autoignitions of diesel fuel/air mixtures behind reflected shock waves. In: editor^editors. the 4th European Combustion Meeting.

[4] D.R. Haylett, D.F. Davidson, R.K. Hanson, Ignition delay times of low-vapor-pressure fuels measured using an aerosol shock tube, Combustion and Flame 159 (2012) 552-561.

[5] S. Gowdagiri, W. Wang, M.A. Oehlschlaeger, A shock tube ignition delay study of conventional diesel fuel and hydroprocessed renewable diesel fuel from algal oil, Fuel 128 (2014) 21-29.

Corresponding author: yangli.nuig@gmail.com